INNOVATECH SOLUTIONS

Phase 1: Design Document

Resilient, Observable, Cost-Efficient Web & Data Infrastructure on AWS

Case Study 1 · Track: Flavor A - Tech Driven (Advanced Network & Infrastructure)

Stretch track: Flavor B - Developer Driven (DevOps & Automation)

# 1.  Introduction

Innovatech Solutions has outgrown manual infrastructure management. This document records the Phase 1 architecture and technology decisions for a new environment that is resilient (self-healing, fault-tolerant), agile (adaptable to future needs), observable (comprehensively monitored), and cost-efficient (optimized for peak usage while minimizing off-peak spend), built to handle a ticket-sales-style traffic pattern.

The environment is provisioned with Infrastructure as Code from the outset. All decisions below map directly to the non-functional requirements REQ-NCA-P1-01 through REQ-NCA-P1-08.

# 2.  Requirements Traceability

Each non-functional requirement is satisfied by a specific design decision, detailed in the sections that follow.

| Requirement | Design Response | Covered in |
| --- | --- | --- |
| REQ-NCA-P1-01 Network Segmentation | AWS hub-and-spoke via Transit Gateway; two isolated spoke VPCs | 3.1 |
| REQ-NCA-P1-02 Secure Resource Access | DB has no public IP or internet route; SSM Session Manager for admin access | 3.1, 3.2 |
| REQ-NCA-P1-03 Webservice Deployment | EC2 provisioned via Terraform, configured via Ansible | 3.2, 4 |
| REQ-NCA-P1-04 Scalability | Auto Scaling Group behind an ALB, scaling on CloudWatch metrics | 3.2 |
| REQ-NCA-P1-05 Observability Architecture | Prometheus + Grafana + Alertmanager, self-hosted | 3.3 |
| REQ-NCA-P1-06 Infrastructure as Code | Terraform (provisioning) + Ansible (configuration) | 4, 6 |
| REQ-NCA-P1-07 CI/CD | GitHub Actions, self-hosted runner | 6 |
| REQ-NCA-P1-08 DevOps Platform | GitHub for source control and deployment tracking | 6 |

# 3.  System Architecture

## 3.1  Network Architecture

The network follows a hub-and-spoke topology built on an AWS Transit Gateway (TGW). The hub is the sole routing point between spokes, giving a single place to enforce and audit traffic flow, and letting a new spoke be attached later without altering existing ones, directly satisfying the REQ-NCA-P1-01 requirement for both isolation and future expansion.

### Spoke VPCs

| VPC | CIDR | Contents & Internet Reachability |
| --- | --- | --- |
| DMZ | 10.0.1.0/24 | Application Load Balancer (public) and the environment's NAT Gateway. Only VPC with a direct Internet Gateway. |
| Internal VPC ( Web subnets A and B) | 10.0.2.0/25 and 10.0.2.128/25 | Auto Scaling Group of EC2 instances running Nginx. No public IP; egress-only internet access via the DMZ NAT (routed through the TGW). |
| Internal VPC (Data subnet) | 10.0.3.0/25 | MySQL. No default route to the internet at all, no NAT, no IGW. Security group permits inbound 3306/tcp from the Web VPC's security group only. |
| Internal VPC (Management subnet) | 10.0.3.128/25 | Prometheus, Grafana, Alertmanager, and the self-hosted GitHub Actions runner. Egress-only via the DMZ NAT. |

### Secure Resource Access (REQ-NCA-P1-02)

- Database isolation: no public IP, no internet-bound route in the route table, and a security group that only accepts traffic from the Web Subnets security group on port 3306, route-table isolation at the network layer, security-group enforcement at the port layer.

- Administrative access: AWS Systems Manager (SSM) Session Manager, via a VPC interface endpoint in each private spoke. No bastion host and no inbound SSH rule anywhere in the environment.

### Routing Approach

- Static TGW routes are the default on every spoke attachment and are sufficient on their own to satisfy REQ-NCA-P1-01's traffic-control requirement.

- BGP is introduced on one attachment: a simulated branch/corporate site connects over a Site-to-Site VPN (customer-gateway side simulated on an EC2 instance running a software VPN daemon), exchanging routes with the TGW dynamically rather than by hand.

## 3.2  Compute & Application Architecture

| Layer | Decision | Justification |
| --- | --- | --- |
| Web server | Nginx | Event-driven, asynchronous architecture handles many concurrent connections with lower memory/CPU overhead than Apache's process/thread model, directly relevant to a ticket-sales traffic spike. |
| Database | MySQL, self-managed on EC2 | MySQL is the most widely deployed open-source relational database, with the largest ecosystem, documentation base, and tooling support, including first-class native support if migrated to managed RDS later. |
| Compute model | EC2 + Ansible (not containers or serverless) | Containers would give faster scale-out but require reworking the deployment model chosen for REQ-NCA-P1-03; serverless (Lambda) is a poor fit for a persistent, always-on web/DB service. EC2 + Ansible keeps effort concentrated on the network/routing depth this project prioritizes. |
| Scalability | Auto Scaling Group behind the ALB, scaling on a CloudWatch CPU/request-count metric | Directly satisfies REQ-NCA-P1-04, instance count grows for peak (ticket on-sale) traffic and shrinks during off-peak periods, so cost tracks actual demand rather than a fixed fleet size. |

Both the web tier (ASG) and, at minimum, the ALB span multiple Availability Zones so that a single AZ failure does not take the site down, a direct response to the business context's resilience goal. The self-managed database currently runs single-AZ; multi-AZ replication for the database is flagged as an open item for Phase 4 resilience testing.

## 3.3  Monitoring & Observability Architecture (REQ-NCA-P1-05)

The deployed stack is Prometheus + Grafana, self-hosted in the Management VPC:

- Data collection: node_exporter on every Web and Data instance, scraped by Prometheus.

- Visualization: Grafana dashboards for host and application metrics.

- Alerting: Alertmanager, configured for high-CPU, service-down, and connectivity-failure conditions.

- Extensibility: adding a new component only requires a new scrape target and exporter — no restructuring of the existing stack. This is deliberate: it supports the Flavor A assignment to automate monitoring so newly added components are monitored adequately from the moment they're deployed.

Commercial tools (Datadog, New Relic, Splunk) are not deployed, they are researched separately as the comparison baseline against this self-hosted stack on features, scalability, operational complexity, cost, and vendor support, as requested in the business context brief. That write-up, and the full metric-by-metric justification for REQ-NCA-P1-05, is produced as its own Monitoring Design deliverable.

# 4.  Technology Choices Summary

| Area | Chosen | Alternatives Considered |
| --- | --- | --- |
| Cloud provider | AWS | Azure, GCP, on-premises (compared in the TCO report) |
| Network topology | Transit Gateway hub-and-spoke, multi-VPC | Single VPC with subnets (rejected — thinner routing story, less isolation) |
| Web server | Nginx | Apache, IIS |
| Database | MySQL (self-managed EC2) | MariaDB, PostgreSQL, RDS-managed MySQL |
| Compute model | EC2 + Auto Scaling Group | ECS/Fargate, Lambda |
| Configuration management | Ansible | Chef, Puppet, cloud-init only |
| IaC tool | Terraform | Pulumi, CloudFormation |
| Monitoring | Prometheus + Grafana (self-hosted) | Datadog, New Relic, Splunk, CloudWatch-native |
| DevOps platform / CI-CD | GitHub + GitHub Actions (self-hosted runner) | GitLab + GitLab CI, Azure DevOps |
| Admin access | SSM Session Manager | Bastion host |

# 5.  DevOps & IaC Setup

- Platform: GitHub hosts the project repository, pull requests, and deployment history (REQ-NCA-P1-08).

- CI/CD: GitHub Actions with a self-hosted runner (a small EC2 instance in the Management VPC) runs two pipelines, application build/test/deploy, and Terraform plan/apply for infrastructure changes (REQ-NCA-P1-07).

- IaC structure: Terraform modules for network (VPCs, TGW, route tables), compute (ASG, launch template, ALB), and monitoring infrastructure; Ansible playbooks for Nginx, MySQL, and exporter configuration (REQ-NCA-P1-06).